ANNEXE:

National Specifications for

Luxembourg

Version 2025-01

Table of contents

Part A:	Specification for passive interception	4

A.1	Basis of this specification	4

A.2	List of abbreviations	6

A.3	Chosen options and amendments	8

A.3.1	Re ETSI TS 101 671 [1] – H I S T O R I C A L -	8

A.3.1.1	Re ETSI TS 101 671 [1], General section	9

A.3.1.2	Re ETSI TS 101 671 [1], Annex A circuit-switched network handover	11

A.3.1.3	Re ETSI TS 101 671 [1], Annex C HI2 delivery mechanisms and procedures	12

A.3.1.4	Re ETSI TS 101 671 [1], Annex E Use of subaddress and calling party number…	12

A.3.1.5	Re ETSI TS 101 671 [1], Annex F GPRS HI3 interface	13

A.3.1.6	Re ETSI TS 101 671 [1], Annex D.5 ASN.1 - description of IRI (HI2)	13

A.3.2	Re 3GPP TS 33.108 [3]	14

A.3.2.1	Re 3GPP TS 33.108 [3], General section	16

A.3.2.2	Re 3GPP TS 33.108 [3], Annex A HI2 delivery mechanisms and procedures	19

A.3.2.3	Re 3GPP TS 33.108 [3], Annex C UMTS and EPS HI3 interface	19

A.3.2.4	Re 3GPP TS 33.108 [3], Annex G Sending of Content of Communication Messages	19

A.3.2.5	Re 3GPP TS 33.108 [3], Annex J Use of subaddress and calling party number…	19

A.3.2.6	Re 3GPP TS 33.108 [3], Annex O LALS Reporting	20

A.3.2.7	Re 3GPP TS 33.108 [3], Annex B Structure of data at the handover interface	20

A.3.3	Re 3GPP TS 33.128 [4]	21

A.3.3.1	Re 3GPP TS 33.128 [4], General Section	22

A.3.4	Re ETSI TS 102 232-1 [5]	24

A.3.4.1	Re ETSI TS 102 232-1 [5], General section	24

A.3.4.2	Re ETSI TS 102 232-1 [5], Annex D IRI by post and pre-processing HI3 information	25

A.3.4.3	Re ETSI TS 102 232-1 [5], Annex F Traffic management of the handover interface	26

A.3.4.4	Supplements to ETSI TS 102 232-1 [5], Annex A ASN.1 syntax trees	26

A.3.5	Re ETSI TS 102 232-2 [6]	27

A.3.5.1	Re ETSI TS 102 232-2 [6], General Section	27

A.3.5.2	Supplements to ETSI TS 102 232-2 [6], Annex D Messaging ASN.1	27

A.3.6	Re ETSI TS 102 232-3 [7]	28

A.3.6.1	Re ETSI TS 102 232-3 [7], General Section	28

A.3.6.2	Supplements to ETSI TS 102 232-3 [7], 8 ASN.1 for IRI and CC	28

A.3.7	Re ETSI TS 102 232-4 [8]	29

A.3.7.1	Re ETSI TS 102 232-4 [8], General Section	29

A.3.7.2	Supplements to ETSI TS 102 232-4 [8], 8 ASN.1 for IRI and CC	29

A.3.8	Re ETSI TS 102 232-5 [9]	30

A.3.8.1	Re ETSI TS 102 232-5 [9], General Section	30

A.3.8.2	Supplements to ETSI TS 102 232-5 [9], 7 ASN.1 specification for IRI and CC	30

A.3.9	Re ETSI TS 102 232-6 [10]	31

A.3.9.1	Re ETSI TS 102 232-6 [10], General Section	31

A.3.9.2	Supplements to ETSI TS 102 232-6 [10], Annex A ASN.1 for IRI and CC	31

A.3.10	Re ETSI TS 102 232-7 [11]	32

A.3.10.1	Re ETSI TS 102 232-7 [11]; General Section	32

A.3.10.2	Supplements to ETSI TS 102 232-7 [11]; Annex A ASN.1 for IRI and CC	32

A.3.11	Re ETSI TS 103 707 [12]	33

A.3.11.1	Supplements to ETSI TS 103 707 [12], Annex C	33

A.4	Technical Provisions	34

A.4.1	ISDN-based transmission – H I S T O R I C A L –	34

A.4.2	IP-based transmission	35

Part B:	Specification for active interception	37

B.1	General Requirements	37

B.2	Technical Provisions	37

Introduction

This document consists of Part A and Part B:

PART A: Specification for passive interception

This part describes the technical implementation of lawful interception of telecommunications. Implementation is carried out based on the relevant ETSI and 3GPP specifications (refer to A.1), and this part describes the options and amendments that have been defined for Luxembourg.

PART B: Specification for active interception

This part describes the support that shall be supplied by the NWO/AP/SvP (Network Operator / Access Provider / Service Provider) in case of operations, which require active interception.

Scope

This document is written in English and will be provided to the NWO/AP/SvP upon request. It applies to any NWO/AP/SvP in the Grand Duchy of Luxembourg that is obligated to comply with lawful interception.

Terminology of modal verbs

In the present document, modal verbs are used as follows:

SHALL: 	Used to express mandatory requirements (provisions that have to be followed)

SHALL NOT: 		Negative form of SHALL

SHOULD: 	Used to express recommendations (provisions that an implementation is expected to follow unless there is a strong reason for not doing so)

SHOULD NOT: 	Negative form of SHOULD

MAY:	MAY is used to express permissions (provisions that an implementation is allowed to follow or not follow)

NEED NOT: 		Negative form of MAY

- Specification for passive interception

  - Basis of this specification

This Part A includes the ETSI and 3GPP documents listed below, which are applicable in the versions noted below or in later versions, and are to be observed.

[1]	ETSI TS 101 671 	V3.15.1	(2018-06): 	Lawful Interception (LI); Handover
	(- H I S T O R I C A L -)					Interface for the lawful interception of
							telecommunications traffic

[2]	ETSI TS 103 280	V2.13.1	(2024-07): 	Lawful Interception (LI); Dictionary for
							common parameters

[3] 	3GPP TS 33.108	V18.0.0	(2024-03):	3rd Generation Partnership Project; Technical
							Specification Group Services and System
							Aspects; 3G security; Handover interface for
							Lawful Interception (LI) (Release 18)

[4] 	3GPP TS 33.128	V18.8.0	(2024-06):	3rd Generation Partnership Project; Technical
							Specification Group Services and System
							Aspects; Security; Protocol and procedures
							for Lawful Interception (LI); Stage 3
							(Release 18)

[5]	ETSI TS 102 232-1	V3.32.1	(2024-07): 	Lawful Interception (LI); Handover Interface
							and Service-Specific Details (SSD) for IP
							delivery; Part 1: Handover specification for
							IP delivery

[6]	ETSI TS 102 232-2	V3.16.1	(2023-03): 	Part 2: Service-specific details for messaging
 						services

[7]	ETSI TS 102 232-3	V3.14.1	(2024-07): 	Part 3: Service-specific details for internet 							access services

[8]	ETSI TS 102 232-4	V3.7.1	(2024-01): 	Part 4: Service-specific details for Layer 2 							services

[9]	ETSI TS 102 232-5	V3.21.1	(2024-07): 	Part 5: Service-specific details for IP
							Multimedia services

[10]	ETSI TS 102 232-6	V3.5.1	(2023-03): 	Part 6: Service-specific details for 								PSTN/ISDN services

[11]	ETSI TS 102 232-7	V3.13.1	(2023-03):	Part 7: Service-specific details for Mobile 							Services

[12]	ETSI TS 103 707	V1.10.1	(2024-12):	Lawful Interception (LI); Handover Interface
							for HTTP delivery

[13]	ETSI TS 103 120	V1.18.1	(2024-12)	Lawful Interception (LI); Interface for
						warrant information*		
* Currently there is no usage of an electronic interface for warrant information in the Grand Duchy of Luxembourg, but it is very likely that it will be mandatory in future version of this specification. New implementations should already be planned considering the need for an electronic warrant interface according to ETSI TS 103 120 [13].

The chosen options and national amendments to these ETSI and 3GPP documents are listed in the following chapters of Part A. If no options or amendments are defined in Part A, the corresponding ETSI or 3GPP document will be applicable without change in the version specified above or in a later version.

  - List of abbreviations

| Abbreviation | Description |
| --- | --- |
| 3GPP | 3rd Generation Partnership Project |
| AP | Access Provider |
| ASCII ASN.1 | American Standard Code for Information Interchange Abstract Syntax Notation One |
| CC | Content of Communication |
| CCLID | CC Link Identifier |
| CS | Circuit Switched |
| CSP | Communication Service Provider |
| DCC | Delivery Country Code |
| DSL | Digital Subscriber Line |
| EIPID | Extended Interception Point Identifier |
| EOL | End of Life |
| EPS | Evolved Packet System |
| ETSI | European Telecommunications Standards Institute |
| FTP | File Transfer Protocol |
| GGSN | Gateway GPRS Support Node |
| GLIC | GPRS LI Correlation |
| GPRS | General Packet Radio Service |
| GSM | Global System for Mobile communications |
| HI1 | Handover Interface 1 |
| HI2 | Handover Interface 2 |
| HI3 | Handover Interface 3 |
| HI4 | Handover Interface 4 |
| ID | Identifier |
| IPID | Interception Point Identifier |
| IPSec | Internet Protocol Security |
| IRI | Intercept Related Information |
| ISDN | Integrated Services Digital Network |
| LAF | Location Acquisition Function |
| LALS | Lawful Access Location Services |
| LARF | Location Acquisition Requesting Function |
| LEA | Law Enforcement Agency |
| LEMF | Law Enforcement Monitoring Facility |
| LI | Lawful Interception |
| LI HIQR | Lawful Interception Handover Interface Query Response |
| LIID | Lawful Interception Identifier |
| NEID | Network Element Identifier |
| NID | Network Identifier |
| NWO | Network Operator |
| PD | Packet Data |
| POI | Point of Interception |
| PS | Packet Switched |
| RFC | Request for Comment |
| ROSE | Remote Operation Service Element |
| RTP | Real-Time Transport Protocol |
| SA | Stand Alone |
| SGSN | Serving GPRS Support Node |
| SIP | Session Initiation Protocol |
| SMS | Short Message Service |
| SSD | Service-Specific Details |
| SvP | Service Provider |
| TCP | Transmission Control Protocol |
| TDM | Time Division Multiplexing |
| TS | Technical Specification |
| UDP | User Datagram Protocol |
| UE | User Equipment |
| ULIC | UMTS LI Correlation |
| UMTS | Universal Mobile Telecommunication System |
| UPS | Uninterruptible Power Supply |
| UUS | User to User Signalling |

  - Chosen options and amendments

    - Re ETSI TS 101 671 [1] – H I S T O R I C A L -

NOTE:

This part is marked as historical and shall not be used for new implementations! New telecommunication networks and services are IP based Packet Switched networks and Packet Switched services. Some legacy networks and services are still Circuit Switched networks and/or services even if the transport might be IP based. New LI implementations shall use the specifications mentioned in A.3.2 – A3.11 of this document.

For CS HI3 (ISDN based) delivery, please refer also to the description of the implemented SIP gateway described in chapter A.4.1.

The NWO/AP/SvP should ensure that the handover takes place from network elements located in the territory of the state of Luxembourg.

Options that can be chosen in each country and amendments to ETSI TS 102 671 [1] are listed in this chapter.

      - Re ETSI TS 101 671 [1], General section

| Re Section | Reference / Description | National provision / extension |
| --- | --- | --- |
| 5.1 | Handover Interface port 1 (HI1) Design, electronic or manual | The HI1 interface will remain manual. If a legal basis is created for electronic implementation of the HI1 interface, this will be introduced at a later stage. Exception: LI management notifications (liActivated, liDeactivated, liModified, alarms-indicator) shall be sent via the electronic HI2 interface (refer to ETSI TS 101 671 [1], D.4). |
| 5.2 | Handover Interface port 2 (HI2) | The IRI records shall be transmitted individually. |
| 6.1 | Lawful Interception Identifier (LIID) | The LIID shall be defined by the LEA. |
| 6.2.1 | Network Identifier (NID) | The NID consists of the Operator ID and Network Element Identifier (NEID). The Operator ID consists of up to 5 characters; the nomenclature is defined and updated by the LEA. The NEID should be set by the NWO/AP/SvP. |
| 7.2 | LI notifications towards the LEMF | LI management notifications (liActivated, liDeactivated, liModified, alarms-indicator) shall be sent via the electronic HI2 interface (refer to ETSI TS 101 671 [1], D.4). |
| 8.1 | Data transmission protocols (HI2) | Only FTP shall be used, ROSE shall not be used. |
| 9 | HI3: Interface port for Content of Communication | The Content of Communication (CC) shall be presented as a transparent, unencrypted copy, if the encryption is managed by the network. Encryption not managed by the network, e.g. user provided end-to-end encryption, need not to be removed by the network. |
| 10.1 | Timing | If IRI cannot be transmitted, they shall be buffered by the NWO/AP/SvP. Minimum buffer time: 3 days. |

| Re Section | Reference / Description | National provision / extension |
| --- | --- | --- |
| 11 | Security aspects | ISDN transmission: For CS HI3 delivery, refer to the implementation specified in chapter A.4.1 of this document. IP-based transmission: For IP based delivery, refer to the implementation specified in chapter A.4.2 of this document. |
| 12 | Quantitative aspects | The following figures can be used as a basis for dimensioning the technical equipment installed at the NWO/AP/SvP: - 50 targets for the first 10000 subscribers - an additional 20 targets for each further 10000 subscribers started (e.g.: NWO with 76000 subscribers shall be able to set up at least 50+7*20= 190 targets) Remark: The dimensioning mentioned above applies only for CS networks. For PD networks, pls. refer to the dimensioning remarks in the clarification preamble of chapter A.3.2, A.3.3 and A.3.4 of this specification. |

      - Re ETSI TS 101 671 [1], Annex A circuit-switched network handover

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| A.1.3 | Use of identifiers | As option A (A.5.4.1) has been specified in A.5.4 in ETSI TS 101 671 [1], the rules according to table A.1.1, left side, apply. |
| A.3.2 | Structure of IRI records | Only IRI conforming to ASN.1-description are permissible. |
| A.3.2.1 | Control information for HI2, item 5 | In existing implementations, Date and time may be transmitted as local time. Remark: New implementations shall use UTC Time as Generalized Time. Existing implementations, if technically feasible, should change to UTC Time as Generalized Time in close coordination and mutually agreed with the LEA. |
| A.4 | HI3: Interface port for Content of Communication | The Content of Communication (CC) shall be presented as a transparent, unencrypted copy, if the encryption is managed by the network. Encryption not managed by the network, e.g. user provided end-to-end encryption, need not be removed by the network. |
| A.4.1 | Delivery of Content of Communication (CC) | Use of UUS1 has been specified. In order to enable subaddressing as fall-back, the LIID for circuit-switched intercepts shall solely be implemented by number (LIID is set by the LEA). |
| A.4.2 | Delivery of packetized Content of Communication (CC) | Text messages (SMS) and UUS shall be transmitted via the HI2 interface. |
| A.4.4.1 | Failure of CC links | The NWO/AP/SvP shall make 3 attempts at an interval of 5 seconds. |
| A.4.4.2 | Fault reporting | Error messages shall be transmitted over HI2 in accordance with ETSI TS 101 671 [1], Annex D.4, if the system used by the NWO/AP/SvP supports this functionality. |

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| A.4.5 | Security requirements at the HI3 interface port | Refer to ETSI TS 101 671 [1], 5.1.1, re 11. Security Aspects |
| A.5.4 | Multi party calls - general principles, options A, B | Option A shall be used only. |
| A.6.4.1 | Explicit call transfer, CC link | Option 2 has been specified. |
| A.6.22 | User-to-User signalling (UUS) | Transmission via HI2 shall be used, also refer to ETSI TS 101 671 [1], A.4.2. |
| A.8.3 | HI3 (delivery of CC) | Correlation information is transmitted in conformance with ETSI TS 101 671 [1], A.4.1. |
| A.8.4 | HI2 (delivery of IRI) | Redundant information shall be sent for each further event. |

      - Re ETSI TS 101 671 [1], Annex C HI2 delivery mechanisms and procedures

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| C.0 | ROSE or FTP | Only FTP shall be used, ROSE shall not be used. |
| C.2.2 | Use of FTP | Method B shall be used. |

      - Re ETSI TS 101 671 [1], Annex E Use of subaddress and calling party number…

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| E.2 | Subaddress options | According to Table E.2.1 in ETSI TS 101 671 [1], the default value for type of subaddress is “user specified”. |
| E.3.2 | Field order and layout | To distinguish between "old" transmission and transmission in accordance with this specification, the octets 16-23 should be allocated as follows: If ‘old’ transmission: no entry If transmitting according to this specification: "Xa.bb.cc" X: E for ETSI a: main version TS 101 671 bb: technical version cc: editorial version (Example: E3.15.01 for TS 101 671 V3.15.1) |

      - Re ETSI TS 101 671 [1], Annex F GPRS HI3 interface

| Re Section | Reference / Description | National provision / extension |
| --- | --- | --- |
| F.1 | Functional architecture | GGSN and SGSN interception shall be set as standard in order to obtain a maximum of information. If for technical reasons only one kind of interception is possible, then SGSN interception shall be set up. |
| F.3 | HI3 Delivery of Content of Communication (CC) | Transmission by GLIC/TCP or FTP/TCP shall be used, GLIC/UDP shall not be used. |
| F.3.2.2 | Use of FTP | Method B shall be used. |
| F.3.2.2 | Use of FTP | The following triggers have been specified: send timeout = 10s volume trigger = 10 MByte |

      - Re ETSI TS 101 671 [1], Annex D.5 ASN.1 - description of IRI (HI2)

Clarification: Any parameter described in the ASN.1 notation, even if marked as OPTIONAL in the ETSI TS, SHALL be transmitted, insofar it exists with regard to the respective message.

| ASN.1-Reference | Reference / Description | National provision / Extension |
| --- | --- | --- |
| 04022.1 | Location | In case of a mobile connection, the following parameters shall be set: - GlobalCellID - GSMLocation or UMTSLocation |
| 04022.1 | Location/GSMLocation/ GeoCoordinates | The AZIMUTH value shall be set except in the case of an omni-directional antenna (360° antenna). |
| 04022.1 | Location/GSMLocation/ GeoCoordinates | The MapDatum shall be WGS84. |
| 04022.1 | partyinformation | An individual partyinformation shall be sent for EACH party involved in a communication. |
| 04022.1 | partyinformation/partyidentity | All existing parameters shall be defined, depending on the means of communication used. |

    - Re 3GPP TS 33.108 [3]

Clarifications:

General quantitative aspects for LI delivery functions:

The NWO/AP/SvP's shall preventively adapt the capacity of the LI delivery function (DF3) in order to cope with the evolution of intercepted traffic. For this purpose, the NWO/AP/SvP's operators shall communicate their DF3 systems bandwidth capacity to the LEA and the LEA shall measure the actual Hi3 interfaces usage per DF3 system by period of 5 minutes. If the value of the 95th percentile over 1 calendar month measurements exceeds 60% of DF3 system maximum capacity, the LEA shall notify the NWOs/APs/SvPs. After receiving such notification, the NWOs/APs/ SvPs shall initiate the actions required to extend the capacity of their system within 30 days.

Location Information:

The MapDatum for any location information shall be WGS84.

NWO/AP/SvP’s current or future networks shall ensure that any UE location information available in the network is reported to the LEA via the LI POIs.

Home Routing Scenarios:

NWO/AP/SvP shall take care, in case traffic of foreign roamers is tunnelled to foreign networks, any encryption introduced in these tunnels is removed before delivery of the Interception Product to the LEA. If no technical standardization is available to comply with this requirement, Home Routing tunnels to foreign networks shall not be encrypted.

Prevention of duplicated delivery:

NWO/AP/SvP shall avoid duplicated delivery of the same intercepted event, if this is technically feasible in state of the art LI infrastructure.

Timestamps:

New implementations shall use UTC Time as Generalized Time, the precision should be 1 microsecond for all Timestamps or, if not available, the highest precision possible.

Existing implementations, if technically feasible, should change to UTC Time as Generalized Time in close coordination and mutually agreed with the LEA.

Transmission method:

3GPP TS 33.128 [4] uses the IP-based transmission method according to ETSI TS 102 232-1 [5] and ETSI TS 102 232-7 [11], in which the data according to 3GPP TS 33.128 is encapsulated. This IP-based transmission method shall also be used for 3GPP TS 33.108. For existing implementations, the change of transmission methods shall be done in close coordination and mutually agreed with the LEA.

Mapping of data to targets:

NWO/AP/SvP shall ensure that mapping of received data to LI targets is possible for all service characteristics (e.g. also for Multi-SIM scenarios).

Size and Duration of CC-Sessions:

If technically feasible and supported by the LI system of the NWO/AP/SvP, CC-Sessions should be closed and reopened without packet loss after 6h or a size of 4GByte per session, whatever occurs first. This method shall not apply to VoIP-sessions.

Continue-IRI:

Apart from the definitions in 3GPP TS 33.108 [3] and if technically feasible, Continue IRIs including updated Location Information should be sent at least every 15 minutes.

The NWO/AP/SvP should ensure that the handover takes place from network elements located in the territory of the state of Luxembourg.

The options that can be chosen in each country and amendments to 3GPP TS 33.108 [3] are listed in this chapter.

      - Re 3GPP TS 33.108 [3], General section

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| 4.4.1 | Handover Interface port 2 (HI2) | The IRI records shall be transmitted individually. |
| 4.5 | HI2: Interface port for intercept related information | If it is not possible to transmit the IRI, they shall be buffered by the NWO/AP/SvP. Minimum buffer time: 3 days. |
| 4.5.1 | Data transmission protocols (HI2) | The usage of FTP is considered historical, FTP transmission shall not be used for new implementations! Existing implementations, if technically feasible, should change to delivery according to ETSI TS 102 232-1 [5] and ETSI TS 102 232-7 [11] in close coordination and mutually agreed with the LEA. New implementations shall use delivery according to ETSI TS 102 232-1 [5] and ETSI TS 102 232-7 [11]. |
| 5.1.2.1 6.1.2 7.1.2 8.1.3 9.1.3 10.1.2 11.1.3 13.1.1.3 14.2.2.3 17.1.2 | Network Identifier (NID) | The NID consists of the Operator ID and Network Element Identifier (NEID). The Operator ID consists of up to 5 characters; the nomenclature is defined and updated by the LEA. The NEID should be set by the NWO/AP/SvP. If set, NEID shall be coded as eight bit ASCII-string. |
| 5.1.5 | Use of identifiers | As option A (5.4.4.1) has been specified in in 3GPP TS 33.108 [3], 5.4.4, the rules according to table 5.1, left side, apply. |
| 5.2.2.1 | Control information for HI2, item 5 | In existing implementations, Date and time may be transmitted as local time. Remark: New implementations shall use UTC Time as Generalized Time. Existing implementations, if technically feasible, should change to UTC Time as Generalized Time in close coordination and mutually agreed with the LEA. |
| 5.2.3 | HI2 (delivery of IRI) | Redundant information shall be sent for each further event. |
| 5.3.1 | CS-based Delivery of Content of Communication (CC) | Use of UUS1 has been specified. In order to enable subaddressing as fall-back, the LIID for circuit-switched intercepts shall solely be implemented by number (LIID is set by the LEA). |
| 5.3.3 | Security requirements at the interface port of HI3 | For HI3 delivery, refer to the implementation specified in chapter A.4.1 of this document. |
| 5.4.4.0 | Multi party calls - general principles, options A, B | Option A shall be used only. |
| 5.5.4.1 | Explicit call transfer, CC link | Option 2 has been specified. |
| 5.5.15 | User-to-User signalling (UUS) | Transmission via HI2 has been specified. |
| 6.2.1 7.2.1 8.2.1 9.2.1 10.2.1 11.2.1 12.2 13.1.2.1 14.2.3.1 | Timing | If IRI cannot be transmitted, they shall be buffered by the NWO/AP/SvP. Minimum buffer time: 3 days. |
| 6.2.1 7.2.1 8.2.1 9.2.1 10.2.1 11.2.1 12.2 13.1.2.1 14.2.3.1 | Precision of timestamps | The precision of timestamps should be 1 microsecond for all Timestamps or, if not available, the highest precision possible. |
| 6.3 7.3 8.3 9.3 10.3 11.3 12.3 13.1.3 14.2.4.1 | Security aspects | IP-based transmission: For IP based delivery, refer to the implementation specified in chapter A.4.2 of this document. |
| 6.4 7.4 8.4 9.4 10.4 11.4 12.4 13.1.4 14.2.5.1 | Quantitative aspects | The following figures can be used as a basis for dimensioning the technical equipment installed at the NWO/AP/SvP: - 50 targets for the first 10000 subscribers - an additional 20 targets for each further 10000 subscribers started (e.g.: NWO with 76000 subscribers shall be able to set up at least 50+7*20= 190 targets) |
| 6.5.0 | UMTS data events | The event “start of interception with mobile station attached” mentioned in 3GPP TS 33.108 [3], Table 6.1 should generate a Report IRI. |

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| 6.5.1.1 | Triggering for the SMS-report record | In the four scenarios where there is a choice depending on national requirements, the SMS Report IRI shall always occur when the G-SGSN receives the notification of successful receipt. |
| 6.5.1.1 | Content field of the SMS-report record | The content field of an SMS-report record shall contain the full PDU according to 3GPP TS 23.040. |
| 6.5.1.1 | REPORT record information | All events marked as national option or as dependent on national regulations shall generate a Report IRI. |
| 6.6 | IRI reporting for packet domain at GGSN | This option does not need to be implemented in Luxembourg. |
| 6.7 | Content of Communication interception for packet domain at GGSN | The option has been chosen. All target traffic available at the interception node shall be routed to the LEA. |
| 10.5.1.1 | REPORT record information | All events marked as national option or as dependent on national regulations shall generate a Report IRI. |
| 12.5 | IRI for IMS-based VoIP | The national option has been chosen, LEMF shall be informed about the unavailability of CC. |
| 16 | Cell Site Reporting | The Cell Site Supplemental Information records from the Cell Site database shall be sent. |

      - Re 3GPP TS 33.108 [3], Annex A HI2 delivery mechanisms and procedures

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| A.0 | HI2 delivery mechanisms and procedures | The usage of FTP is considered historical, FTP transmission shall not be used for new implementations! Existing implementations, if technically feasible, should change to delivery according to ETSI TS 102 232-1 [5] and ETSI TS 102 232-7 [11] in close coordination and mutually agreed with the LEA. New implementations shall use delivery according to ETSI TS 102 232-1 [5] and ETSI TS 102 232-7 [11]. |
| A.2.2 | Use of FTP | Method B shall be used. |
| A.2.2 | Use of FTP | The following triggers have been specified: send timeout = 10s volume trigger = 10MByte |

      - Re 3GPP TS 33.108 [3], Annex C UMTS and EPS HI3 interface

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| C.0 | UMTS and EPS HI3 interfaces; methods of transmission | Only ULICv1 via TCP stream shall be used. |
| C.2.2 | Use of FTP | Method B shall be used. |

      - Re 3GPP TS 33.108 [3], Annex G Sending of Content of Communication Messages

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| G.3.1.1.3 | Keep-alives | Keep-alives shall be used. |

      - Re 3GPP TS 33.108 [3], Annex J Use of subaddress and calling party number…

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| J.2.3.2 | Field order and layout | To distinguish between "old" transmission and transmission in accordance with this specification, the octets 16-23 should be allocated as follows: If ‘old’ transmission: no entry If transmitting according to this specification: "Xa.bb.cc" X: E for ETSI a: main version TS 101 671 bb: technical version cc: editorial version (Example: E3.15.01 for TS 101 671 V3.15.1) |

      - Re 3GPP TS 33.108 [3], Annex O LALS Reporting

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| O | LALS Reporting | The NWO/AP/SvP shall inform LEA if LALS is supported in NWO/AP/SvP’s network. In this case, LALS shall be activated for specific targets upon LEA’s request. The required parameters will be defined by the LEA. |

      - Re 3GPP TS 33.108 [3], Annex B Structure of data at the handover interface

Clarification: Any parameter described in the ASN.1 notation that is delivered with 3GPP TS 33.108 [3], even if marked as OPTIONAL, SHALL be transmitted, insofar it exists with regard to the respective message.

| ASN.1 -Reference | Reference / Description | National provision / Extension |
| --- | --- | --- |
| 04022.49 | Eps-HI3-PS | To avoid any doubt: The timestamp parameter in the ULIC header shall be provided. |

    - Re 3GPP TS 33.128 [4]

Until detailed supplements will be defined in later versions of this specification, interceptions based on 3GPP TS 33.128 [4] shall be established in close coordination and mutually agreed with the LEA. The general definitions for the IP based delivery described in chapter A.4.2 of this document apply also for this kind of delivery.

Clarifications:

General quantitative aspects for LI delivery functions:

The NWO/AP/SvP's shall preventively adapt the capacity of the LI delivery function (DF3) in order to cope with the evolution of intercepted traffic. For this purpose, the NWO/AP/SvP's operators shall communicate their DF3 systems bandwidth capacity to the LEA and the LEA shall measure the actual Hi3 interfaces usage per DF3 system by period of 5 minutes. If the value of the 95th percentile over 1 calendar month measurements exceeds 60% of DF3 system maximum capacity, the LEA shall notify the NWOs/APs/SvPs. After receiving such notification, the NWOs/APs/ SvPs shall initiate the actions required to extend the capacity of their system within 30 days.

Location Information:

The MapDatum for any location information shall be WGS84.

NWO/AP/SvP’s current or future networks shall ensure that any UE location information available in the network is reported to the LEA via the LI POIs.

Home Routing Scenarios:

NWO/AP/SvP shall take care, in case traffic of foreign roamers is tunnelled to foreign networks, any encryption introduced in these tunnels is removed before delivery of the Interception Product to the LEA. If no technical standardization is available to comply with this requirement, Home Routing tunnels to foreign networks shall not be encrypted.

Pseudonymized IDs:

To increase privacy, in 5G and subsequent network generations, IDs should be routed through the network using pseudonyms that do not allow any conclusions to be drawn about the participants of the communication. The LI implementations shall ensure that these pseudonymized IDs, at the LI-related Handover Interfaces, can be assigned to a specific user again. This basic requirement also applies to roaming scenarios.

Prevention of duplicated delivery:

NWO/AP/SvP shall avoid duplicated delivery of the same intercepted event, if this is technically feasible in state of the art LI infrastructure.

Timestamps:

New implementations shall use UTC Time as Generalized Time, the precision should be 1 microsecond for all Timestamps or, if not available, the highest precision possible.

Existing implementations, if technically feasible, should change to UTC Time as Generalized Time in close coordination and mutually agreed with the LEA.

Mapping of data to targets:

NWO/AP/SvP shall ensure that mapping of received data to LI targets is possible for all service characteristics (e.g. also for Multi-SIM scenarios).

Size and Duration of CC-Sessions:

If technically feasible and supported by the LI system of the NWO/AP/SvP, CC-Sessions should be closed and reopened without packet loss after 6h or a size of 4GByte per session, whatever occurs first. This method shall not apply to VoIP-sessions.

Continue-IRI:

Apart from the definitions in 3GPP TS 33.128 [4] and if technically feasible, Continue IRIs including updated Location Information should be sent at least every 15 minutes.

The NWO/AP/SvP should ensure that the handover takes place from network elements located in the territory of the state of Luxembourg.

      - Re 3GPP TS 33.128 [4], General Section

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| 4.3 | Basic principles for external handover interfaces | The HI4 interface described in 3GPP TS 33.128 [4] shall be implemented by the NWO/AP/SvP. |
| 4.4.4 | Location Reporting | The usage of location reporting type is not allowed. The POIs and MDF2/MDF3 shall report location every time the target location information is received at the POI (including location update with no physical change of location). |
| 5.5 | Extended Interception Point Identifier (EIPID) | To support error handling in LEMF, the EIPID field shall be populated. |
| 5.7 | Protocols for LI_HIQR | NWO/AP/SvP with domestic 5G SA (Stand Alone) implementations shall support the interface described in 3GPP TS 33.128 [4], 5.7 Protocols for LI_HIQR. The implementation of the interface including the connection to the LEA shall be established in close coordination and mutually agreed with the LEA. |
| 7.3 | Location | Either Lawful Access Location Services (LALS) according to 3GPP TS 33.128 [4], chapter 7.3.1 or Location Acquisition according to 3GPP TS 33.128 [4], chapter 7.3.5 shall be implemented. The implementation of the interface including the connection to the LEA shall be established in close coordination and mutually agreed with the LEA. |
| 7.3 | Location | Cell Database Reporting according to 3GPP TS 33.128 [4], chapter 7.3.2 should be implemented, if technically feasible. The implementation of the interface including the connection to the LEA shall be established in close coordination and mutually agreed with the LEA. |

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| 7.3 | Location | Separated Location Reporting according to 3GPP TS 33.128 [4], chapter 7.3.4 should be implemented, if technically feasible. The implementation of the interface including the connection to the LEA shall be established in close coordination and mutually agreed with the LEA. |

    - Re ETSI TS 102 232-1 [5]

Clarifications:

Quantitative aspects for LI delivery function in PD networks:

The NWO/AP/SvP's shall preventively adapt the capacity of the LI delivery function (DF3) in order to cope with the evolution of intercepted traffic. For this purpose, the NWO/AP/SvP's operators shall communicate their DF3 systems bandwidth capacity to the LEA and the LEA shall measure the actual Hi3 interfaces usage per DF3 system by period of 5 minutes. If the value of the 95th percentile over 1 calendar month measurements exceeds 60% of DF3 system maximum capacity, the LEA shall notify the NWOs/APs/SvPs. After receiving such notification, the NWOs/APs/SvPs shall initiate the actions required to extend the capacity of their system within 30 days.

Size and Duration of CC-Sessions:

If technically feasible and supported by the LI system of the NWO/AP/SvP, CC-Sessions should be closed and reopened without packet loss after 6h or a size of 4GByte per session, whatever occurs first. This method shall not apply to VoIP-sessions.

Continue-IRI:

Apart from the definitions in ETSI TS 102 232-1 [5] and if technically feasible, Continue IRIs including updated Location Information should be sent at least every 15 minutes.

MTU-Size:

The MTU-Size will be mutually agreed between NWOs/APs/SvPs and shall not exceed 1.500 Byte. Jumbo Frames shall not be used.

The NWO/AP/SvP should ensure that the handover takes place from network elements located in the territory of the state of Luxembourg.

The options that can be chosen in each country and amendments to ETSI TS 102 232-1 [5] are listed in this chapter.

      - Re ETSI TS 102 232-1 [5], General section

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| 5.2.3 | Authorization country code | Specified as "LU". |
| 5.2.4 | Communication identifier | The Operator ID consists of up to 5 characters; the nomenclature is defined and updated by the LEA. The NEID should be set by the NWO/AP/SvP. If set, NEID shall be coded as eight bit ASCII-string. The DCC parameter shall be set. |

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| 5.2.6 | Payload timestamp | The timeStampQualifier shall be set. Re Note 2: New implementations shall use UTC Time as Generalized Time, the precision should be 1 microsecond for all Timestamps or, if not available, the highest precision possible. Existing implementations, if technically feasible, should change to UTC Time as Generalized Time in close coordination and mutually agreed with the LEA. Remark: Timestamps shall always provide the highest accuracy possible. |
| 5.2.11 5.2.13 | Interception Point Identifier (IPID) / Extended Interception Point Identifier (EIPID) | To support error handling in LEMF, either the IPID or the EIPID field shall be populated. |
| 6.2.3 | Aggregation of payloads | Combined transmission of IP packets is authorised, but shall not delay transmission for more than 2 seconds. |
| 6.2.4 | Sending a large block of application-level data | Segmentation is not used. |
| 6.2.5 | Padding data | Padding is not used. |
| 6.2.6 | Payload Encryption | Payload encryption is not used. |
| 6.3.1 | General | TCP/IP socket connections are used. |
| 6.3.2 | Opening and closing connections | The NWO/AP/SvP shall make 3 connection attempts at an interval of 10 seconds. |
| 6.3.4 | Keep-alives | Keep-alives shall be used. |
| 6.3.5 | Option negotiation | Option negotiation is currently not used, but may be implemented on LEAs request at a later stage. |
| 6.4.2 | TCP Settings | The port numbers to be used will be specified by the LEA. |
| 6.4.3 | Acknowledging data | Option 1 is chosen. |
| 7.2 | Security requirements | IP-based transmission: For IP based delivery, refer to the implementation specified in chapter A.4.2 of this document. |
| 7.2.3 | Integrity | The NWO/AP/SvP shall perform integrity checks. The example parameters according to ETSI TS 102 232-1 [5], chapter J.1 should be used. The NWO/AP/SvP shall set the hashAlgorithm ASN.1 field to inform LEA about the hash algorithm used. |

      - Re ETSI TS 102 232-1 [5], Annex D IRI by post and pre-processing HI3 information

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| D.4 | IRI by post and pre-processing HI3 information | Pre-processing at LEMF to generate IRI is not considered, the IRI shall be generated by post-processing at CSP’s domain. |

      - Re ETSI TS 102 232-1 [5], Annex F Traffic management of the handover interface

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| F.4 | National considerations | Filtering at the mediation function should only be implemented upon explicit request by the LEA. |
| F.5.2 | Maximum buffering time | To protect against loss of data due to equipment or network problems, the buffering time shall be 15 minutes taking into account the maximum bandwidth at the network interface of the delivery function. Remark: The buffering time may be increased in future versions of this specification. |

      - Supplements to ETSI TS 102 232-1 [5], Annex A ASN.1 syntax trees

Clarification: Any parameter described in the ASN.1 notation, even if marked as OPTIONAL in the ETSI TS, SHALL be transmitted, insofar it exists with regard to the respective message.

| ASN.1-Reference | Reference / Description | National Provision / Extension |
| --- | --- | --- |
| 04022.51 | General | The provisions in ETSI TS 102 232-1 [5] remain unchanged. |

    - Re ETSI TS 102 232-2 [6]

      - Re ETSI TS 102 232-2 [6], General Section

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| 4.2 | Unified messaging | Handover of intercepted e-mail shall be according to EmailCC and EmailIRI structures. |
| 7 | E-mail attributes | All attributes mentioned in ETSI TS 102 232-2 [6], 7.1 to 7.10 shall be set. |

      - Supplements to ETSI TS 102 232-2 [6], Annex D Messaging ASN.1

Clarification: Any parameter described in the ASN.1 notation, even if marked as OPTIONAL in the ETSI TS, SHALL be transmitted, insofar it exists with regard to the respective message.

| ASN.1-Reference | Reference / Description | National Provision / Extension |
| --- | --- | --- |
| 04022.52 | General | The provisions in ETSI TS 102 232-2 [6] remain unchanged. |

    - Re ETSI TS 102 232-3 [7]

      - Re ETSI TS 102 232-3 [7], General Section

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| 6.2.1.2 | Handover of NAT translated IP addresses | In the described scenarios, publicly exposed IP address that is known to be associated with the target via NAT translation shall be provided using the otherTargetIdentifiers field. The considerations in ETSI TS 102 232-3 [7], Annex E shall be respected. |
| 6.2.2 | Use of location field | The location parameter shall be set. |
| 7.2.A | HI3 truncated attributes | On LEA’s explicit request, the optional truncation mechanism shall be activated, either for specific interception measures or for all interception. In this case, the number of octets shall be mutually agreed between LEA and NWO/AP/SvP. |

      - Supplements to ETSI TS 102 232-3 [7], 8 ASN.1 for IRI and CC

Clarification: Any parameter described in the ASN.1 notation, even if marked as OPTIONAL in the ETSI TS, SHALL be transmitted, insofar it exists with regard to the respective message.

| ASN.1-Reference | Reference / Description | National Provision / Extension |
| --- | --- | --- |
| 04022.53 | General | The provisions in ETSI TS 102 232-3 [7] remain unchanged. |

    - Re ETSI TS 102 232-4 [8]

      - Re ETSI TS 102 232-4 [8], General Section

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| 7.3 | HI3 truncated attributes | On LEA’s explicit request, the optional truncation mechanism shall be activated, either for specific interception measures or for all interception. In this case, the number of octets shall be mutually agreed between LEA and NWO/AP/SvP. |

      - Supplements to ETSI TS 102 232-4 [8], 8 ASN.1 for IRI and CC

Clarification: Any parameter described in the ASN.1 notation, even if marked as OPTIONAL in the ETSI TS, SHALL be transmitted, insofar it exists with regard to the respective message.

| ASN.1-Reference | Reference / Description | National Provision / Extension |
| --- | --- | --- |
| 04022.54 | General | The provisions in ETSI TS 102 232-4 [8] remain unchanged. |

    - Re ETSI TS 102 232-5 [9]

      - Re ETSI TS 102 232-5 [9], General Section

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| 5.2.3 | Location information | The location information shall be reported. |
| 5.6 | Direction for IMS IRI for Signalling Messages | The payloadDirection parameter shall be used. The values fromTarget, toTarget and indeterminate shall be used as described. The values combined and notapplicable shall not be used. |
| 5.7.1 | Direction for SIP sessions | The sessionDirection parameter shall be used. The values fromTarget, toTarget, combined and indeterminate shall be used as described. The value notapplicable shall not be used. |

      - Supplements to ETSI TS 102 232-5 [9], 7 ASN.1 specification for IRI and CC

Clarification: Any parameter described in the ASN.1 notation, even if marked as OPTIONAL in the ETSI TS, SHALL be transmitted, insofar it exists with regard to the respective message.

| ASN.1-Reference | Reference / Description | National Provision / Extension |
| --- | --- | --- |
| 04022.55 | General | The provisions in ETSI TS 102 232-5 [9] remain unchanged. |

    - Re ETSI TS 102 232-6 [10]

      - Re ETSI TS 102 232-6 [10], General Section

REMARK: 	If the NWO/AP/SvP’s equipment provides delivery of CC via dedicated ISDN channels as described and defined in ETSI TS 101 671 [1], please refer to the implementation specified in chapter A.4.1 of this document.

For RTP based delivery according to ETSI TS 102 232-6 [10], CC shall be coded in G.711.

| Re Section | Reference / Description | National provision / Extension |
| --- | --- | --- |
| 6.3.2 | Supplementary information | All fields mentioned in the table shall be set. |

      - Supplements to ETSI TS 102 232-6 [10], Annex A ASN.1 for IRI and CC

Clarification: Any parameter described in the ASN.1 notation, even if marked as OPTIONAL in the ETSI TS, SHALL be transmitted, insofar it exists with regard to the respective message.

| ASN.1-Reference | Reference / Description | National Provision / Extension |
| --- | --- | --- |
| 04022.56 | General | The provisions in ETSI TS 102 232-6 [10] remain unchanged. |
| 04022.56 | FrameType | The FrameType artificialRtpFrame shall not be used. |

    - Re ETSI TS 102 232-7 [11]

      - Re ETSI TS 102 232-7 [11]; General Section

The provisions in the specified documents remain unchanged.

      - Supplements to ETSI TS 102 232-7 [11]; Annex A ASN.1 for IRI and CC

Clarification: Any parameter described in the ASN.1 notation, even if marked as OPTIONAL in the ETSI TS, SHALL be transmitted, insofar it exists with regard to the respective message.

    - Re ETSI TS 103 707 [12]

Until detailed supplements will be defined in later versions of this specification, interceptions based on ETSI TS 103 707 [12] shall be established in close coordination and mutually agreed with the LEA. The general definitions for the IP based delivery described in chapter A.4.2 of this document apply also for this kind of delivery.

The delivered objects shall be according to ETSI TS 103 120 [13] using JSON format.

      - Supplements to ETSI TS 103 707 [12], Annex C

The models A, B and C described in ETSI TS 103 707 [12], Annex C.2,C.3 and C.4 are allowed, the usage shall be mutually agreed with the LEA.

  - Technical Provisions

    - ISDN-based transmission – H I S T O R I C A L –

NOTE:

This part is marked as historical, ISDN based transmission shall not be used for new implementations! Existing implementations, if technically feasible, should change to IP based delivery of CC in close coordination and mutually agreed with the LEA.

The classical HI3 delivery via ISDN CS lines towards the LEMF is not supported anymore, as ISDN based subscriber lines are EOL. To support legacy LI implementations that still use HI3 delivery via CS, the LEA implemented a CS/SIP-Gateway in one of the NWOs.

The delivery towards the CS/SIP-Gateway can be realized in two ways:

Option A: CS-Delivery (TDM) towards the CS/SIP-Gateway:

The NWO/AP/SvP will use the telephone number of the CS/SIP-Gateway as termination address of the CS-delivery. The conversion of Called Party Subaddress and Calling Party Subaddress to SIP will be done automatically in the CS/SIP-Gateway using RFC 3966.

Option B: SIP-Delivery towards the CS/SIP-Gateway (On request and only after LEA’s approval):

NWO/AP/SvP that do not support standard LI PD delivery or that don’t support the delivery via the handover Gateways defined in chapter A.4.2 of this document, may use the CS/SIP-Gateway to deliver SIP traffic to the LEMF.

In this case, the NWO/AP/SvP delivering towards the CS/SIP-Gateway is responsible to do the conversion of Called Party Subaddress and Calling Party Subaddress to SIP using RFC 3966.

    - IP-based transmission

The delivery of IP-based content can be realized in two ways:

For NWO/AP/SvP, that have or use network equipment on Luxemburgish territory or that provide identifiers attributable to the State of Luxembourg (for example but not limited to: Telephone numbers starting with +352 according to E.164 numbering plan, domain names ending with top level domain *.lu):

In order to comply with national security requirements, the LEA established handover gateways in several domestic data centres. NWO/AP/SvP that have or use network equipment in Luxembourg shall use one of the data centres as transfer point.

The technical details of the handover as well as the most updated list of available data centres will be provided by the LEA.

Security from handover gateway to the LEMF is in the responsibility of the LEA, security from NWO/AP/SvP’s network towards the handover gateway is in the responsibility of the NWO/AP/SvP.

The technical implementation of an optional encryption between NWO/AP/SvP’s network and the LEA’s handover gateway will be mutually agreed between NWO/AP/SvP and LEA. If technically feasible, the NWO/AP/SvP’s shall inform the LEA about the possible encryption options and the encryption tunnel used for this optional encryption should be terminated at the LEA’s handover gateway.

For all network connections under its responsibility, the NWO/AP/SvP shall use state of the art methods and technology to comply with the general requirements about authenticity, confidentiality and integrity according to relevant ETSI and 3GPP standards referenced in A.1 of this document.

NWO/AP/SvP shall cover all expenses for the network connection towards the handover gateway, including the connection to the handover point inside the data centre premises.

For all other NWO/AP/SvP:

The delivery between NWO/AP/SvP and LEA will be mutually agreed between NWO/AP/SvP and LEA. Both parties will assure that security requirements according to the relevant ETSI and 3GPP standards referenced in A.1 of this document are met using state of the art methods and technology.

NWO/AP/SvP shall cover all expenses for equipment on its side towards the handover connection (e.g. if data will be transmitted via public Internet, all equipment and functionalities required between its network and the Internet connection shall be covered by the NWO/AP/SvP).

- Specification for active interception

  - General Requirements

In accordance with the relevant domestic laws, a NWO/AP/SvP shall support the integration of active interception equipment into its network upon request by the LEA.

The active interception equipment will be provided and operated by the LEA responsible.

Depending on the case and the nature of the active interception, the point and type of integration into the NWO/AP/SvP’s network and the level of required support may vary.

Prior to the integration, the LEA responsible will communicate the detailed requirements to the NWO/AP/SvP.

  - Technical Provisions

The required technical provisions will be announced by the LEA on a case-by-case basis. The general infrastructural requirements will be the same as described in chapter A.4.2 of this document.

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) hi2(1) version18(18)}

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulintercept(2) threeGPP(4) hi3eps(9) r17(17) version-0(0)} or later

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) li-ps(5) genHeader(1) version38(38)} or later

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) li-ps(5) email(2) version20(20)} or later

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) li-ps(5) iPAccess(3) version18(18)} or later

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) li-ps(5) l2Access(4) version9(9)} or later

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) li-ps(5) iPMultimedia(5) version17(17)} or later

{itu-t(0) identified-organization(4) etsi(0) securityDomain(2) lawfulIntercept(2) li-ps(5) pstnIsdn(6) version6(6)} or later